# Practical -4

## AIM: Setup and configure a LAN (Local area network) using a Switch and

## Ethernet cables in your lab.

# What is a LAN?

A Local Area Network (LAN) refers to a network that connects devices within a limited area,

such as an office building, school, or home. It enables usersto share resources, including data,

printers, and internet access. LAN connects devices to promote collaboration and transfer

information between users,such as computers, printers,servers, and switches. A local area

network (LAN) switch serves as the primary connecting device, managing and directing

communications within the local network. Each connected device on a LAN switch can

communicate directly with each other, allowing for fast and secure data transfer.

# How to set up a LAN

Step 1. Plan and Design an appropriate network topology taking into account network

requirements and equipment location.

Step 2. You can take 4 Computers, a Switch with 8, 16, or 24 ports which issufficient for

networks of these sizes, and 4 Ethernet cables.

Step3: Connect your computers to network switch via an Ethernet cable, which is as simple as

plugging one end of the Ethernet cable into your computer and the other end into your network

switch.

Step4: Assign IP address to your PCs

1. Log on to the client computer as Administrator or as Owner.

2. Click Network and Internet Connections.

3. Right Click Local Area Connection/Ethernet->Go to Properties->Select Internet

Protocol (TCP/IPv4)->Click on Properties->Select use the following ip address option

and assign ipaddress.

Similarly assign IP addressto all the PCS connected to switch.

PC1-IP address: 10.1.1.1, subnet mask 255.0.0.0

PC2-IP address-10.1.1.2, subnet mask 255.0.0.0

PC3-IP address 10.1.1.3, subnet mask 255.0.0.0.

PC4-IP address 10.1.1.4, subnet mask 255.0.0.0.

Step 5:- Configure a network switch:

1. Connect your computer to the switch: To accessthe switch's web interface, you will need

to connect your computer to the switch using an Ethernet cable.

2. Log in to the web interface: Open a web browser and enter the IP address of the switch

in the address bar. This should bring up the login page for the switch's web interface.

Enter the username and password to log in.

3. Configure basic settings: Once you're logged in, you will be able to configure basic

settings for the switch,

4. Assign IP address as: 10.1.1.5, subnet mask 255.0.0.0.

Step 6:- Check the connectivity between switch and other machine by using ping command in

the command prompt of the device.

Step 7: Select a folder, ->go to properties-> click Sharing tab->share it with everyone on the

same LAN.

Step 8. Try to access the shared folder from others Computers of the network.